Overview

- Python Script: Reads keyboard inputs and sends corresponding commands to the Arduino.

- Arduino Code: Generates a PPM signal based on the commands received from the Python script. The PPM signal is then sent to the INAV flight controller.

- INAV Configuration: Set up to receive PPM signals from the Arduino, allowing for manual control via the keyboard.

Hardware Setup

- Arduino: Connect the PPM output pin to the PPM input on your INAV-compatible flight controller.

- 2.4 GHz Transceiver (optional): If you want wireless control, you can add a transceiver like the nRF24L01 to send inputs wirelessly to the Arduino.

- Flight Controller: Configure it in INAV Configurator to accept PPM input.

Arduino Code

This code generates a PPM signal based on the inputs received from the Python script.

cpp

Copy code

#define PPM_PIN 9  // PPM output pin

#define NUM_CHANNELS 6  // Number of channels (Throttle, Yaw, Pitch, Roll, Aux1, Aux2)

int channels[NUM_CHANNELS] = {1500, 1500, 1500, 1500, 1000, 1000};  // Initial channel values (1500us center)

void setup() {

pinMode(PPM_PIN, OUTPUT);

digitalWrite(PPM_PIN, HIGH);

Serial.begin(9600);  // Start serial communication

}

void loop() {

if (Serial.available() > 0) {

String input = Serial.readStringUntil('\n');

input.trim();

if (input.startsWith("THROTTLE")) {

channels[0] = input.substring(9).toInt();

} else if (input.startsWith("YAW")) {

channels[1] = input.substring(4).toInt();

} else if (input.startsWith("PITCH")) {

channels[2] = input.substring(6).toInt();

} else if (input.startsWith("ROLL")) {

channels[3] = input.substring(5).toInt();

} else if (input == "SERVO_TOGGLE") {

channels[4] = (channels[4] == 1000) ? 2000 : 1000;  // Toggle between 1000 and 2000

}

sendPPM();

}

}

void sendPPM() {

const int frameLength = 22500;  // 22.5ms for 50Hz PPM signal

int pulseLength = 300;  // Length of the sync pulse

for (int i = 0; i < NUM_CHANNELS; i++) {

pulseLength += channels[i];  // Add channel pulse length

}

int syncLength = frameLength - pulseLength;  // Calculate the sync pulse length

for (int i = 0; i < NUM_CHANNELS; i++) {

// Send the channel pulse

digitalWrite(PPM_PIN, LOW);

delayMicroseconds(300);  // 300us pulse

digitalWrite(PPM_PIN, HIGH);

delayMicroseconds(channels[i] - 300);  // Remainder of the channel time

}

// Send the sync pulse

digitalWrite(PPM_PIN, LOW);

delayMicroseconds(300);  // 300us pulse

digitalWrite(PPM_PIN, HIGH);

delayMicroseconds(syncLength - 300);  // Remainder of the sync time

}

Python Code

This Python script reads keyboard inputs and sends commands to the Arduino to update the PPM channels.

python

Copy code

import serial

import time

import keyboard

# Open serial port (Replace 'COM4' with the port your Arduino is connected to)

ser = serial.Serial('COM4', 9600, timeout=1)

# Wait for serial connection to be established

time.sleep(2)

# Initial values for drone control

throttle = 1500  # Start at midpoint (1000-2000)

yaw = 1500

pitch = 1500

roll = 1500

servo_toggled = False

def send_command(command_type, value):

command = f"{command_type} {value}\n"

ser.write(command.encode('utf-8'))

print(f"Sent command: {command.strip()}")

response = ser.readline().decode('utf-8').strip()

print(f"Arduino response: {response}")

def toggle_servo():

global servo_toggled

servo_toggled = not servo_toggled

command = "SERVO_TOGGLE\n"

ser.write(command.encode('utf-8'))

print(f"Sent command: {command.strip()}")

response = ser.readline().decode('utf-8').strip()

print(f"Arduino response: {response}")

try:

print("Use W/S for throttle, A/D for yaw, I/K for pitch, J/L for roll, and R to toggle the servo.")

while True:

if keyboard.is_pressed('w'):

throttle += 10

if throttle > 2000:

throttle = 2000

send_command("THROTTLE", throttle)

time.sleep(0.1)

if keyboard.is_pressed('s'):

throttle -= 10

if throttle < 1000:

throttle = 1000

send_command("THROTTLE", throttle)

time.sleep(0.1)

if keyboard.is_pressed('a'):

yaw -= 10

if yaw < 1000:

yaw = 1000

send_command("YAW", yaw)

time.sleep(0.1)

if keyboard.is_pressed('d'):

yaw += 10

if yaw > 2000:

yaw = 2000

send_command("YAW", yaw)

time.sleep(0.1)

if keyboard.is_pressed('i'):

pitch += 10

if pitch > 2000:

pitch = 2000

send_command("PITCH", pitch)

time.sleep(0.1)

if keyboard.is_pressed('k'):

pitch -= 10

if pitch < 1000:

pitch = 1000

send_command("PITCH", pitch)

time.sleep(0.1)

if keyboard.is_pressed('j'):

roll -= 10

if roll < 1000:

roll = 1000

send_command("ROLL", roll)

time.sleep(0.1)

if keyboard.is_pressed('l'):

roll += 10

if roll > 2000:

roll = 2000

send_command("ROLL", roll)

time.sleep(0.1)

if keyboard.is_pressed('r'):

toggle_servo()

time.sleep(0.3)  # Debounce to prevent multiple toggles

except KeyboardInterrupt:

print("Program terminated")

finally:

ser.close()

Steps to Set Up and Configure with INAV

- Arduino Setup:

  - Connect the Arduino's PPM_PIN (pin 9 in this example) to the PPM input of your INAV flight controller.

  - Upload the Arduino code to your board.

- INAV Configurator Setup:

  - Connect to the Flight Controller: Use the INAV Configurator to connect to your flight controller.

  - Set Receiver Mode: Go to the "Receiver" tab and set the receiver mode to PPM.

  - Calibrate Channels: Ensure that all channels (Throttle, Yaw, Pitch, Roll, Aux1 for the servo) are working correctly by moving the corresponding keys and observing the response in INAV.

  - Set Up Modes: In the "Modes" tab, configure any necessary modes (e.g., Arm/Disarm, GPS Hold, etc.) using the auxiliary channels.

- Testing:

  - Manual Control: Test manual control using the keyboard inputs to ensure that throttle, yaw, pitch, and roll are responsive.

  - Servo Toggle: Press R to toggle the servo between 0° and 180°. Verify that this works in INAV as well.

  - GPS Preloaded Flights: Test your preloaded GPS missions to ensure that the manual controls do not interfere with autonomous flight operations.

- Fine-Tuning:

  - Adjust channel ranges or response curves in INAV Configurator if needed.

  - Ensure that manual control inputs do not conflict with INAV's autonomous modes, especially when switching between manual and GPS modes.

Final Notes:

- Safety: Always test these controls in a safe environment before attempting real flights.

- Advanced Protocols: If you want to implement more advanced features or better integration, consider using a more complex protocol like SBUS or MSP, but this will require more sophisticated coding.

- Expandability: You can expand this system by adding more auxiliary channels for additional functionality, such as camera control or additional servos.